| Exp No: 2 | Study of Network cables - straight-through and crossover cable |
| --- | --- |

AIM:

To study the different types of network cables and to make a straight-through cable and a cross-wired (crossover) cable using a crimping tool.

APPARATUS REQUIRED:

- Cat5e / Cat6 UTP cable (about 1 metre for each cable)

- RJ45 connectors (4 numbers)

- Crimping tool with built-in cutter and stripper

- LAN cable tester (main unit and remote unit)

ALGORITHM / PROCEDURE:

# Part A - Study of network cables

- Observe the sample twisted pair, coaxial and fibre optic cables available in the lab.

- Note the outer jacket marking (for example Cat5e, 24 AWG) and the number of pairs.

- Write down the use, speed and maximum length of each cable type in the table given below.

# Part B - Making the cables

- Cut the UTP cable to the required length (about 1 m) using the cutter.

- Remove about 2.5 cm of the outer jacket from both ends with the stripper. Take care not to cut the inner wires.

- Untwist the four pairs and straighten the eight wires with fingers.

- For the straight-through cable: arrange the wires in T568B order on both ends.

- For the crossover cable: arrange T568B order on one end and T568A order on the other end.

- Cut the wire tips even so that about 1.3 cm of the wires is left outside the jacket.

- Hold the RJ45 connector with the clip facing down, push the wires inside till they touch the end of the connector. The jacket should also go inside a little.

- Put the connector in the crimping tool and press firmly so that the pins cut into the wires.

- Repeat steps 2 to 8 for the other end of the cable.

- Insert both ends into the LAN tester and check the LED sequence.

# Types of network cables

| Cable |  |  |  | Connector |  |  |  | Max. length |  |  |  | Speed |  |  |  |  | Used for |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Twisted pair (UTP / STP) |  |  |  | RJ45 |  |  |  | 100 m |  |  |  | 10 Mbps to 10 Gbps |  |  |  |  | LAN, connecting PCs to switch |  |  |
| Coaxial (RG-6, RG-59) |  |  |  | BNC / F-type |  |  |  | 185 m to 500 m |  |  |  | 10 Mbps (old Ethernet) |  |  |  |  | Cable TV, CCTV, old LANs |  |  |
| Fibre optic (single / multi mode) |  |  |  | SC / LC / ST |  |  |  | 2 km (multi) up to 40 km+ (single) |  |  |  | 1 Gbps to 100 Gbps |  |  |  |  | Backbone, long distance links |  |  |
| UTP category |  |  |  |  | Max. speed |  |  |  |  |  |  |  |  | Bandwidth |  |  |  |  |  |
| Cat5 |  |  |  |  | 100 Mbps |  |  |  |  |  |  |  |  | 100 MHz |  |  |  |  |  |
| UTP category |  |  |  |  | Max. speed |  |  |  |  |  |  |  |  | Bandwidth |  |  |  |  |  |
| Cat5e |  |  |  |  | 1 Gbps |  |  |  |  |  |  |  |  | 100 MHz |  |  |  |  |  |
| Cat6 |  |  |  |  | 1 Gbps (10 Gbps up to 55 m) |  |  |  |  |  |  |  |  | 250 MHz |  |  |  |  |  |
| Cat6a |  |  |  |  | 10 Gbps |  |  |  |  |  |  |  |  | 500 MHz |  |  |  |  |  |
| Pin | 1 | 2 |  |  |  | 3 |  |  | 4 | 5 |  |  | 6 |  | 7 |  |  | 8 |  |
| T568A | W/G | G |  |  |  | W/O |  |  | B | W/B |  |  | O |  | W/Br |  |  | Br |  |
| T568B | W/O | O |  |  |  | W/G |  |  | B | W/B |  |  | G |  | W/Br |  |  | Br |  |

W = white stripe, G = green, O = orange, B = blue, Br = brown

| Cable | Wiring | Used to connect |
| --- | --- | --- |
| Straight-through | T568B to T568B | PC to Switch, PC to Hub, Router to Switch (unlike devices) |
| Crossover | T568B to T568A | PC to PC, Switch to Switch, Hub to Hub, PC to Router (like devices) |

DIAGRAM:

RJ45

-

End A

RJ45

-

End BRJ45

-

End A

RJ45

-

End B

1

1

2

2

3

3

4

4

5

5

6

6

1

1

2

2

3

3

4

4

5

5

6

6

RJ45

-

End A

RJ45

-

End BRJ45

-

End A

RJ45

-

End B

1

1

2

2

3

3

4

4

5

5

6

6

1

1

2

2

3

3

4

4

5

5

6

6

| 7 |  | 7 | 7 |  | 7 |
| --- | --- | --- | --- | --- | --- |
| 8 |  | 8 | 8 |  | 8 |

| OUTPUT: Colour order of the wires at both ends |  |
| --- | --- |
| End A | End B |

Fig 1: Straight-through cable (pin 1 to 1, 2 to 2 ...) 	Fig 2: Crossover cable (1 to 3, 2 to 6, 3 to 1, 6 to 2)

W/O

O

W/G B

W/B G

W/Br

Br

## Cable 1: Straight-through cable (T568B - T568B)

End A 	End B

W/G

G

W/O B

W/B O

W/Br

Br

Cable 2: Crossover cable (T568B - T568A)

# LAN cable tester result

LEDs glow in the order 3-6-1-4-5-2-7-8 : crossover cable is OK

| Cable | Main unit LED | Remote unit LED | Status |
| --- | --- | --- | --- |
| Straight-through | 1 2 3 4 5 6 7 8 | 1 2 3 4 5 6 7 8 | Working |
| Crossover | 1 2 3 4 5 6 7 8 | 3 6 1 4 5 2 7 8 | Working |

RESULT:

Thus the different types of network cables were studied. A straight-through cable and a crossover cable were made using the crimping tool and both were tested successfully with the LAN tester.